# VISIOBALL

User Guide & Operating Manual

Mobile app for the VisioBall smart training ball  |  App version 1.0.0  |  Firmware v2.6.0  |  Last updated June 2026

What is Visioball? Visioball has a phone app that connects to a smart training ball over Bluetooth. The ball contains an ESP32 microcontroller that plays light/sound sequences. From your phone you can connect to the ball, pick one of five built-in melodies, put the ball to sleep, wake it, and disconnect. This guide explains how to install the app, turn the ball on, connect, use every screen, and keep it updated.

## 1. What's in the box / what you need

- The VisioBall - the physical ball with an ESP32 board, speaker, and battery inside.

- An Android phone - Android 8 or newer with Bluetooth. (iPhone is supported by the code but builds are currently produced for Android.)

- The Visioball app - installed as an APK (see Section 2).

- A USB type C cable - only needed for charging the ball and for developers flashing firmware.

## 2. Installing the app

### Option A - Install the ready-made APK (normal users)

- Copy the visiobal.apk file to your Android phone (via USB, email, or a download link).

- Open the file on the phone. Android will ask permission to "install from this source" - allow it.

- Tap Install. When it finishes, the Visioball icon appears in your app drawer.

- Open the app and allow the Bluetooth and Location permissions when prompted (Android requires location permission for Bluetooth scanning - no location data is collected).

### Option B - Build and run from source (developers)

You need Node.js 18+ and the Android SDK installed.

| Step | Command |
| --- | --- |
| Clone the repository | git clone <repo-url> |
| Enter the folder | cd visiobal |
| Install dependencies | npm install |
| Run on a connected Android phone | npm run android |
| Start the dev server only | npm start |

Important: The free "Expo Go" app does not support Bluetooth. You must use a real development build (npm run android / expo run:android) on a physical phone for the ball to connect. Bluetooth does not work on the web version or on emulators.

## 3. Turning the ball on

- Charge it first if it's new or flat - connect the ball's USB port to a charger.

- Power on the ball (flip its power switch). The ESP32 boots in about a second.

- On power-up the ball immediately starts advertising over Bluetooth under the name VisioBal and is ready to accept a connection. It also begins playing its current melody through the on-board speaker.

The ball is always "discoverable" while powered on and not already connected to a phone. There is no pairing button and no PIN - the app finds it by its name and a private service ID.

## 4. Connecting your phone to the ball

- Make sure the phone's Bluetooth is ON.

- Open the Visioball app. It shows a short splash screen while it loads.

- If you've connected before: the app tries to silently reconnect to the last ball during the splash screen. If it succeeds you go straight to the Control screen - no scanning needed.

- First time (or if the ball isn't found): the app opens the Scan screen and searches for ~8 seconds. Any ball named VisioBal appears in the list, strongest signal first.

- Tap the ball in the list to connect. When connected, a green "Connected" status appears and you move to the controls.

Tip: The app remembers the last ball you used, so next time it usually reconnects on its own.

## 5. Using the app - screen by screen

### Home (Landing) screen

The main hub. You'll see:

- A large 3D model of the ball - drag it with your finger to spin it.

- A music player card with the current track name, and previous / play-pause / next buttons.

- A volume slider and a Radar Scan button.

- A quick menu for all other settings

### Control screen

The detailed control hub, shown after connecting:

- 3D ball at the top - drag to rotate; a soft glow follows it.

- Connection status chip (green = connected).

- Battery indicator (bottom-right of the ball): colour-coded - green 60-100%, yellow 30-60%, red below 30%. It reads the live battery level reported by the ball when available.

- Volume slider and previous / play-pause / next transport buttons.

- A pull-up Quick Menu sheet at the bottom with tiles for Audio, Locate (radar), and Settings, plus a Sleep Mode toggle.

- A Disconnect button to cleanly drop the connection.

### Sound library screen

- Lists all 5 built-in tracks. Tap any track to select it and start it on the ball.

- The big player card shows the selected track, a progress bar, transport buttons, and a volume slider.

- An animated equaliser shows on the active track while playing.

Play/pause and track selection are synchronised between the Home screen and the Sound library - stop on one and it stops on the other; skip to the next track on one and the other updates too.

### Radar screen

An animated radar sweep used as a visual "locate / searching" effect for the ball.

### Settings screen

- Appearance - switch between Dark and Light theme. Your choice is saved.

- Language - English, Dutch, French, German, or Bulgarian. Saved across restarts.

- Notifications - connection alerts and battery warnings.

- Links to Privacy Policy, Terms of Service, and Firmware History.

- Reset - clears saved preferences and the remembered ball.

## 6. The five built-in melodies

Each track in the app maps to one melody stored in the ball's firmware:

| # | Track name | Style | Command sent to ball |
| --- | --- | --- | --- |
| 1 | C Major Journey | Ambient | SONG1 |
| 2 | E Minor Groove | Electronic | SONG2 |
| 3 | Zen State | Meditation | SONG3 |
| 4 | Ocean Waves | Nature | SONG4 |
| 5 | Ambient Pulse | Electronic | SONG5 |

## 7. How the app controls the ball (how it works)

The app and ball talk over Bluetooth Low Energy (BLE). The app is the "client", the ball (ESP32) is the "server". Communication uses one private service and one characteristic the app writes text commands into:

| Setting | Value |
| --- | --- |
| Ball BLE name | VisioBal |
| Command service UUID | 4fafc201-1fb5-459e-8fcc-c5c9c331914b |
| Command characteristic UUID | beb5483e-36e1-4688-b7f5-ea07361b26a8 |
| Battery characteristic UUID | a1b2c3d4-1111-2222-3333-abcdefabcdef |

The flow when you tap a button:

- You tap, e.g., a track in the Sound library.

- The app turns that into a text command (e.g. SONG3).

- The command is checked against a safety whitelist - only known commands are allowed; anything else is rejected before it is ever sent.

- The app writes the command to the ball's command characteristic over BLE.

- The ESP32 receives it, switches to that melody, and keeps playing it through its speaker.

### Commands the app can send

| Command | Meaning | Acted on by current firmware? |
| --- | --- | --- |
| SONG1-SONG5 | Switch to that melody | Yes |
| SLEEP | Put the ball into low-power light sleep | Yes |
| WAKE | Wake the ball from sleep | Yes (any BLE write also wakes it) |
| PLAY / PAUSE / NEXT / PREV | Transport controls | Reserved - the app sends them and updates its own buttons; the current firmware does not act on them yet |
| VOL:0–VOL:100 | Set volume | Reserved - on the ball, volume is set by a physical knob (potentiometer) |
| FREQ:<hz> | Set pitch/frequency | Reserved - on the ball, pitch is set by a physical knob |
| ON / OFF / PING | Light on/off, keep-alive | Reserved for future firmware |

Honest note on volume & pitch: In the current ball firmware the volume and pitch are read from two physical knobs on the ball, not from the app. The app's volume slider and the VOL:/FREQ: commands are wired and validated, ready for a future firmware version that reads them over BLE. Track selection and sleep/wake work fully today.

### Battery, reconnect & sleep

- Battery: while connected, the app reads the ball's battery characteristic about every 1.5 seconds and updates the colour-coded indicator.

- Auto-reconnect: the ID of the last ball is saved on the phone. On the next launch the app tries to reconnect silently (with a 5-second timeout) before falling back to the scan screen.

- Sleep mode: sending SLEEP puts the ESP32 into light sleep to save power; it notifies the phone, then any later BLE write (or WAKE) brings it back and restarts advertising.

## 8. How the ball itself works (hardware)

- Brain: an ESP32 dev board (esp32doit-devkit-v1).

- Sound: generated live by the Mozzi audio synthesis library and output over I2S to a small DAC/speaker. Each melody is a list of notes, durations, and a harmony layer played by two oscillators (triangle + square wave) shaped with an ADSR envelope.

- Controls on the ball: two potentiometers - one for volume, one for pitch/frequency.

- Bluetooth: the ESP32 runs a BLE server, advertises as VisioBal, and listens on its command characteristic for the text commands above.

- Firmware project: built with PlatformIO (Arduino framework). Key files: src/main.cpp (audio + melodies) and src/BLEManager.cpp (Bluetooth + sleep).

## 9. Updating

### Updating the app

- Get the newer visiobal.apk.

- Install it over the top of the existing app (same steps as Section 2A) - your saved theme, language, and remembered ball are kept.

- Developers: git pull, then npm install (in case dependencies changed), then npm run android.

### Updating the ball firmware

Firmware lives in the separate visiobal-hardware repository and is flashed over USB with PlatformIO:

- Open the hardware project in VS Code with the PlatformIO extension.

- Connect the ESP32 to the computer over USB.

- Click Upload (or run pio run -t upload). PlatformIO compiles and flashes the board.

- Open the Serial Monitor at 115200 baud to see connection and command logs.

The in-app Settings → Firmware History screen lists what changed in each firmware version (v2.1.0 through v2.6.0).

## 10. Troubleshooting

| Problem | Try this |
| --- | --- |
| Ball doesn't appear in the scan list | Confirm the ball is powered on; make sure it isn't already connected to another phone; turn the phone's Bluetooth off and on; pull down to scan again. |
| "Bluetooth is turned off" | Enable Bluetooth in the phone's quick settings, then retry. |
| Permissions denied | Go to the phone's app settings for Visioball and allow Bluetooth (nearby devices) and Location. |
| Connects then drops | Move closer to the ball; check the ball's battery; reconnect from the scan screen. |
| No sound | Check the ball's physical volume knob; confirm a track is selected; confirm the ball is awake (not in Sleep Mode). |
| App won't connect at all (developer build) | Make sure you are using a development build, not Expo Go, and that you ran npm run android on a real device. |

## 11. Privacy & safety

- The app makes no network calls - nothing leaves your phone.

- The only data saved on the phone is your theme, language, and the ID of the last ball.

- Commands are validated against a whitelist before being sent, so malformed commands never reach the ball.

## 12. Quick reference

| Item | Value |
| --- | --- |
| App package name | com.marto8090.visiobal |
| App version | 1.0.0 |
| Firmware version | v2.6.0 |
| Ball BLE name | VisioBal |
| App repository | visiobal (this project) |
| Firmware repository | visiobal-hardware (PlatformIO / ESP32) |
| Supported languages | EN, NL, FR, DE, BG |

Visioball is a student prototype, provided as-is for learning and demonstration. Features may be incomplete.