# MoSCoW Prioritisation - Visioball

The MoSCoW method is used to prioritise the requirements of the Visioball project. This method divides features into four categories:

- Must have – essential features required for the product to function

- Should have – important features that improve usability and quality

- Could have – optional enhancements if time and resources allow

- Won’t have – ideas that are outside the scope of the current project

The prioritisation is based on the client requirements, the limitations of the current prototype, and the feasibility of implementing the features within the project timeframe.

# Must have

These requirements are essential for the Visioball to function and to meet the needs of visually impaired users.

- Ball produces sound while moving and while stationary

- Sound is clear, consistent, and loud enough to locate the ball

- Adjustable volume

- Phone control for core functions

- Bluetooth connection between the app and the ball

- Rechargeable battery

- USB-C charging

- Inner core fits within 100 mm diameter

- Ball remains under the maximum weight requirement

- Electronics are securely mounted and shock resistant

- Ball is durable for drops, collisions, and rough play

- Ball reaches at least IP44-level protection

Mobile app must include:

- On/off control

- Volume adjustment

- Sound selection

- Battery status indicator

These requirements address the main problems of the current prototype, which include fixed sound output, lack of phone connectivity, limited durability, and insufficient protection of internal components.

# Should have

These features are important improvements but the system can still function without them in the first version.

- Multi-speaker output for near-360° sound

- Sound should be non-annoying during long use

- Improved internal layout of electronics

- Ball can be opened and repaired without damaging components

- Charging status indicator

- Battery life of 3–6 hours

- Maximum safe sound level (~85 dB)

These features improve usability, reliability, and long-term maintenance of the Visioball.

# Could have

These are additional enhancements that improve the user experience but are not required for the core functionality.

- Lights for partially sighted users

- Battery low indicator

- Different sound after inactivity

- Protected cap over the power switch

- Charging-complete indicator

- Fewer but higher-quality speakers if testing shows six speakers are unnecessary

These features may be added if time and resources allow during development.

# Won’t have (for this project scope)

These ideas were considered but are outside the scope of the current project.

- Motorized self-moving ball

- Heat sensors for detecting catches

- Wireless charging

These features would significantly increase the complexity and development time of the system and are therefore excluded from the current project phase.